产品规格书

PRODUCT SPECIFICATION

客户名称：

CUSTOMER：

产品名称：

PRODUCT　 　NAME：              贴片保险丝   SMD FUSE

型号规格：

MODEL DETAIL：12H (RoHS Compliant)  250mA-10A 63V抗浪涌型(High Inrush Current Withstand)

料  号：

PART NO.：                              12H1500C

供货商：                           深圳市良胜电子有限公司

SUPPLIER：                SHENZHEN LANSON ELECTRONICS CO., LTD.

地  址：                      深圳市宝安区福永富桥第三工业区A18栋

ADDRESS：   A18th Bldg, Fuqiao 3rd Industrial Zone, Fuyong Town. Baoan District, Shenzhen.

| 编 制 PREPARED BY | 审 核 CHECKED BY | 批 准 APPROVED BY |  | 承 认 SIGNATURE | 审 核 CHECKED BY | 批 准 APPROVED BY |
| --- | --- | --- | --- | --- | --- | --- |
| 供方签章： SUPPLIER： 日期： DATE：2019.02.21 |  |  |  | 需方签章： CUSTOMER： 日期： DATE： |  |  |

| SHENZHEN LANSON ELECTRONICS CO., LTD. | PRODUCT SPECIFICATION | EDITION NO. | A/5 |
| --- | --- | --- | --- |
|  |  | FILE NO. | # LS/C-005-34 |
|  | MODEL: 12H | PAGE | PAGE 1 OF 5 |

- 适用范围Scope of Application

本产品适用于各种电子电路中起过电流保护作用。

This product is suitable for various kinds of electronic machines to work for over current protection.

2. 相关标准及认证Relevant Standards and Certification

2.1执行标准Standard: UL248-1、UL248-14

2.2认证Certification

| 认证 Certification | 认证范围 Scope of Certification | 证书编号 Certification No. |
| --- | --- | --- |
| UL | 100mA~30A | E221465 |

3. 结构尺寸及原材料明细Structure Size and Material Detail

3.1 结构尺寸Structure Size (unit: mm)

①

②

③

④

⑤

0.65±0.2

3.2±0.2

1.6±0.2

0.5±0.2

3.2 原材料明细Material Detail

| 编号 No. | 零件名称Part Name | 材质Material |
| --- | --- | --- |
| ① | 电极Electrode | 银Silver |
| ② | 涂层Coating | 玻璃浆Glass cover |
| ③ | 端头End Termination | 镀银Silver plated |
| ④ | 熔丝Fuse element | 银Silver |
| ⑤ | 基片Substrate | 陶瓷Ceramic |

| SHENZHEN LANSON ELECTRONICS CO., LTD. | PRODUCT SPECIFICATION | EDITION NO. | A/5 |
| --- | --- | --- | --- |
|  |  | FILE NO. | # LS/C-005-34 |
|  | MODEL: 12H | PAGE | PAGE 2 OF 5 |

4. 标示Mark

4.1保险丝上需有下列标示:

The fuse shall have the following marking：

| 额定电流 Ampere Rating | 产品标示 Marking |
| --- | --- |
| 250mA | .25 |
| 375mA | E |
| 500mA | 0.5 |
| 750mA | .75 |
| 1A | H |
| 1.5A | K |
| 2A | N |
| 2.5A | O |
| 3A | P |
| 3.5A | R |
| 4A | S |
| 4.5A | X |
| 5A | T |
| 6A | F |
| 7A | 7 |
| 8A | M |
| 10A | U |

| SHENZHEN LANSON ELECTRONICS CO., LTD. | PRODUCT SPECIFICATION | EDITION NO. | A/5 |
| --- | --- | --- | --- |
|  |  | FILE NO. | # LS/C-005-34 |
|  | MODEL: 12H | PAGE | PAGE 3 OF 5 |

5. 电气特性Electrical Characteristics

5.1实验条件: 所有测试环境温度为25±5℃。

Test Condition: All electrical characteristics tests are conducted at an ambient temperature of 25±5℃.

5.2时间-电流特性

Time-Current Characteristics

| 额定电流 Ampere Rating | 额定电流的% % of Ampere Rating | 熔断时间 Opening Time |
| --- | --- | --- |
| 0.1A~10A | 100% | 4 hours Min(大于4小时) |
| 0.1A~10A | 250% | 5 S Max(小于5秒) |
| 0.1A~10A | 300% | 0.1 S~3 S |
| 0.1A~10A | 1000% | 0.2 ms~20 ms |

5.3额定分断能力：

Interrupting Rating:

50 Amperes at 125V DC (0.1A~5A)

50 Amperes at 63V DC (0.1A~5A)

100 Amperes at 32V DC (6A~10A)

6. 环境特性Environmental Characteristics

6.1使用温度范围Operating Temperature

-40℃~125℃。

6.2保存条件Storage Conditions

相对湿度≤75%，可以存放3年。

Relative humidity≤75%, store 3 years.

7. 可靠性试验Reliability Test

7.1冷热冲击试验Thermal Shock Test

MIL-STD-202G, Method 107G。

| SHENZHEN LANSON ELECTRONICS CO., LTD. | PRODUCT SPECIFICATION | EDITION NO. | A/5 |
| --- | --- | --- | --- |
|  |  | FILE NO. | # LS/C-005-34 |
|  | MODEL: 12H | PAGE | PAGE 4 OF 5 |

7.2防潮湿试验Moisture Resistance Test

MIL-STD-202G, Method 106G。

7.3振动试验Vibration Test

MIL-STD-202G, Method 201A。

7.4可焊性试验Solderability Test

MIL-STD-202G, Method 208H, IEC60127-2, IEC60068-2。

8. 时间-电流曲线图Time-Current Curve

| SHENZHEN LANSON ELECTRONICS CO., LTD. | PRODUCT SPECIFICATION | EDITION NO. | A/5 |
| --- | --- | --- | --- |
|  |  | FILE NO. | # LS/C-005-34 |
|  | MODEL: 12H | PAGE | PAGE 5 OF 5 |

9. 焊锡参数Soldering Parameters

9.1 回流焊Reflow Slodering

9.1.1 温度Temperature  260℃

9.1.2 时间Time        ≤10 seconds

9.1.3 最大可以承受三次回流焊

Capable for three times reflow

9.2 波峰焊Wave Soldering

9.2.1 温度Temperature  260℃

9.2.2 时间Time        ≤10 seconds

9.3 手工焊锡Hand Soldering

9.3.1 温度Temperature  380℃

9.3.2 时间Time        ≤3 seconds

10. 包裝Packing

10.1自动机器包装3000PCS/卷。Automaton taped 3000PCS per reel.

10.2包装尺寸Packing Size（unit: mm）